EE 5313 MICROPROCESSOR PROJECT

SPRING 2016

SDRAM CONTROLLER DESIGN

By,

1001227227  -   Navjot S. Saran

1001227227  -   Kshitija Kulkarni

1001227274  -   Mitali Salunkhe

CONTENTS:

- Introduction

- Interfacing Diagram

- Initialization

- Row and Bank Activation

- Reading from SDRAM memory

  - Read with Auto Precharge

  - Read state transistion table (Auto Precharge)

  - Read with Manual Precharge

  - Read state transistion table (Auto Precharge)

- Writing to SDRAM memory

  - Write with Auto Precharge

  - Write state transistion table (Auto Precharge)

  - Write with Manual Precharge

  - Write state transistion table (Auto Precharge)

- Auto Refresh

- Address Signal Generation

- Command Signals

- Control Signals

- Extra Credits

INTRODUCTION:

80386DX has only asynchronous memory support. In this project we are basically designing a SDRAM controller that interfaces the MT48LC8M8A2 SDRAM (2M X 8 X 4banks) memory with 80386DX microprocessor. The SDRAM controller located between the processor and memory and enables the conversion of 80386 commands into synchronous SDRAM memory command and control words. A burst length of 4 is supported by this design. In the project, we have used -75 MT48LC8M8A2 SDRAM Configuration. Our 80386DX microprocessor is working at 25MHz frequency (CLK2 = 50 MHz).

Fig 1. Shows the basic interference diagram between the 80386DX processor, the SDRAM controller and SDRAM memory.

INTERFACING DIAGRAM:

Fig 1: Basic Block diagram of SDRAM – 80386DX Interface

MEMORY INITIALIZATION:

NOP1 = trcd               NOP3 = trp             NOP5 = Start Auto Precharge               NOP7 = 100 us state              NOP9 = tmrd

NOP2 = tcl                  NOP4 = twr             NOP6 = Clock Stable                                      NOP8 = Auto Refresh Delay

Fig 2: State diagram: FSM initialization

Initialization Transition Table

| Current State | Conditions | Next State |
| --- | --- | --- |
| X | Vdd,Vddq,CKE=0,CLK Stable | NOP6 |
| NOP6 | Count_100μs=0 | Precharge |
| Precharge | X | NOP3 |
| NOP3 | Count_trp=0 | Auto-Refresh |
| Auto Refresh | X | NOP8 |
| NOP8 | Count_trfc=0 | Auto-Refresh |
| Auto-Refresh | X | NOP8 |
| NOP8 | Count_trfc=0 | LMR |
| LMR | X | NOP8 |
| NOP8 | Count_tmrd=0 | INIT DONE |

Load Mode Register Fields:

- Burst Length (BL) = 4                                              : 0 1 0

- Burst Type (BT)    = Sequential                             : 0

- CAS Latency          = 2                                             : 0 1 0

- Op Mode              = Standard Operation             : 0 0

- WB                        = Programmed Burst Length : 0

- Reserved                                                                 : 0 0

READ OPERATION WITH AUTO PRECHARGE:

Fig 3: State diagram: Reading from SDRAM memory (with auto precharge)

READ WITH AUTO PRECHARGE TRANSISTION TABLE:

| PRESENT STATE | CONDITION | NEXT STATE |
| --- | --- | --- |
| ACTIVE | LOAD VALUE tRCD | NOP1 |
| NOP1 | Count_tRCD = 0 | READ |
| READ | LOAD VALUE tCL | NOP2 |
| NOP2 | tCL VALUE = 0 | Rxfer1 |
| Rxfer1 | X | Rxfer2 |
| Rxfer2 | X | Rxfer3 |
| Rxfer3 | X (next clock) | Rxfer4 |
| Rxfer3 | X(same clock) | AUTO PRECHARGE (A10 = 1) |
| AUTO PRECHARGE | Load_count=tRP | NOP3 |
| NOP3 | Count_tRP = 0 | IDLE |

Fig 4: State Table: Read with Auto Precharge

READ OPERATION WITH MANUAL PRECHARGE:

Fig 5: State diagram: Reading from SDRAM memory (with manual precharge)

READ WITH MANUAL PRECHARGE TRANSISTION TABLE:

Fig 6: State Table: Read with Manual Precharge

| PRESENT STATE | CONDITION | NEXT STATE |
| --- | --- | --- |
| ACTIVE | LOAD VALUE tRCD | NOP1 |
| NOP1 | tRCD VALUE = 0 | READ |
| READ | LOAD VALUE tCL | NOP2 |
| NOP2 | tCL VALUE = 0 | Rxfer1 |
| Rxfer1 | X | Rxfer2 |
| Rxfer2 | X | Rxfer3 |
| Rxfer3 | X (next clock) | Rxfer4 |
| Rxfer3 | X(same clock) | PRECHARGE (A10 = 0) |
| PRECHARGE | Load_count=tRP | NOP3 |
| NOP3 | tRP VALUE = 0 | IDLE |

WRITE OPERATION WITH AUTO PRECHARGE:

Fig 7: State diagram: Writing to SDRAM memory (with auto precharge)

WRITE WITH AUTO PRECHARGE TRANSISTION TABLE:

| PRESENT STATE | CONDITION | NEXT STATE |
| --- | --- | --- |
| ACTIVE | LOAD VALUE tRCD | NOP1 |
| NOP1 | tRCD VALUE = 0 | WRITE , |
| WRITE | X(same clock) | Wxfer1 |
| Wxfer1 | X(next clock) | Wxfer2 |
| Wxfer2 | X | Wxfer3 |
| Wxfer3 | X | Wxfer4 |
| Wxfer4 | LOAD VALUE tWR | NOP4 |
| NOP4 | tWR VALUE = 0 | NOP5 |
| NOP5 | LOAD VALUE tRP | NOP3 |
| NOP3 | tRP VALUE = 0 | IDLE |

Fig 8: State Transition Table: Write with auto precharge

WRITE OPERATION WITH MANUAL OPERATION:

Fig 9: State diagram: Writing to SDRAM memory(with manual precharge)

WRITE WITH MANUAL PRECHARGE TRANSISTION TABLE:

| PRESENT STATE | CONDITION | NEXT STATE |
| --- | --- | --- |
| ACTIVE | LOAD VALUE tRCD | NOP1 |
| NOP1 | tRCD VALUE = 0 | WRITE , |
| WRITE | X(same clock) | Wxfer1 |
| Wxfer1 | X(next clock) | Wxfer2 |
| Wxfer2 | X | Wxfer3 |
| Wxfer3 | X | Wxfer4 |
| Wxfer4 | LOAD VALUE tWR | NOP4 |
| NOP4 | tWR VALUE = 0 | PRECHARGE |
| NOP5 | LOAD VALUE tRP | NOP3 |
| NOP3 | tRP VALUE = 0 | IDLE |

Fig 10: State Transition Table: Write with manual precharge

AUTO REFRESH:

- This command must be issued each time a refresh is required. All active banks must be precharged before issuing an auto refresh command.

- Address bits are “Don’t Care” during AUTO REFRESH since the internal refresh controller does the addressing part.

- Regardless of the SDRAM memory configuration, there are 4096 rows. Hence it requires 4096 auto refresh cycles every 64ms (commercial and industrial) and 16ms (automotive).

NOTE: SELF REFRESH command is initiated like AUTO REFRESH command except CKE is disabled (LOW).

Fig 11: State Diagram: Auto Refresh

AUTO REFRESH TRANSISTION TABLE:

| PRESENT STATE | CONDITION | NEXT STATE |
| --- | --- | --- |
| IDLE | X | AUTO REFRESH |
| AUTO REFRESH | LOAD VALUE tRFC | NOP |
| NOP | tRFC VALUE = 0 | IDLE |

Fig 12: State Transition Diagram: Auto Refresh

Address Signal Generation:

Fig 13: Block Diagram for Address Generation

In this memory address latching with microprocessor is done as following: A2-A8 forms column address (LSBs of column are hardcoded), A9-A20 forms row address and A21-A22 forms BA0-BA1 signals to the memory.

Data Path Module (For Read):

Fig 14:  Block Diagram of Data Flow (READ)

In data line interface, while reading the data from the memory is latched byte by byte to 4 different latches corresponding to 4 different data for BL=4 and is outputted simultaneously from all 4 latches on READY# signal to give 32-bit data to the microprocessor.

Data Path Module (For write):

Fig 15:  Block Diagram of Data Flow (WRITE)

Similarly, while writing each eight bit- data is given to the memory using a 4:1 multiplexer.

Command Signals:

Depending upon our state we are enabling various command signals such as:

NOP:

LMR (LOAD MODE REGISTER):

ACTIVE:

READ:

Write:

Precharge:

Refresh:

CONTROL SIGNALS:

- RAS ( Row Address Strobe):

- CAS ( Column Address Strobe):

- WE ( Write Enable):

- MRDC (Memory Read Control)#:

80386DX does not have MRDC# and MWTC# signals so we need to generate these signals using these logics.

- MWTC (Memory Write Control)#:

- READY SIGNAL

Design of Various Delay Counters:

- Count to be loaded = 100 μs / 10 ns = 2710H

- Auto Refresh Counter:

trefresh =  64 ms / (No of rows)

µs

= 64 ms / 4096

= 15.6        (Count to be loaded 0820H)

Clk

trcd = 15 ns / 10 ns = 2 clock cycles

- tcl = 15 ns / 10 ns = 2 clock cycles

- trp = 15 ns / 10 ns = 2 clock cycles

- twr

- LMR: tmrd = 15 ns / 7.5 ns = 2 clock cycles

- trfc = 66 / 10  = 7 clock cycles

- trc =  60/10 = 6 clock cycles

- tLMR = 2trfc + trp

EXTRA CREDITS:

Showing Support for MT48LC16M4A2 – 4 Meg x 4 x 4 banks with higher burst length (BL=8).

Here, we will interface a 4bit width memory with 80386DX microprocessor.

Showing Data-Path Module(For Read Operation):

Data Path Module (For Write Operation):

Finite State Diagram:

- Read Cycle:

- Finite State Machine (Write Cycle):

Address Generation(for X 4 mode) :